# Project FALCON Sensor Power and Visibility Approval List

Printable panel handout for revised compact event driven buoy proposal

Prepared for proposal validation | 11 October 2026 | Prices must be reconfirmed before purchase

This document consolidates the required sensors, power components, and night visibility equipment for the revised Project FALCON compact buoy. The purpose is to help the panel approve what will be reused, what must still be bought, and which optional modules can be deferred. The recommended direction is a low cost ESP32 buoy that samples locally, sends summary packets, and uses solar assisted battery power.

Main recommendation: approve the field ready low cost validation package, but buy in stages. Confirm the water pressure sensor first, then validate the power budget using the ESP32, internet module, GPS, wind sensor, pressure interface, battery, solar panel, and night light current draw.

# Recommended approval package

| Package | Includes | Estimated equipment cost | Decision |
| --- | --- | --- | --- |
| Minimum bench validation | ESP32, pressure interface, simulated inputs, GPS, INA219, Wi-Fi, LEDs/buttons from existing kit | PHP 4,700-7,900 plus existing kit parts | Good for proposal and bench testing |
| Field ready low cost validation | Core sensors plus battery, solar panel, solar charger/BMS, LTE if needed, marine night beacon, waterproofing, buoy body, chain, anchor | PHP 8,000-16,000+ depending battery, solar and LTE choices | Best approval target |
| Safer marine option | Better pressure transmitter, industrial wind sensor, LiFePO4 battery, IP68 beacon, better waterproof connectors | PHP 15,000-30,000+ depending quotations | Use only if budget allows |

# Equipment cards for panel approval

Each card shows the role, the current planning price, where to verify the item, and the approval note. Product pictures are real reference photos from online product listings or product photo sources; the exact SKU and seller must still be checked again before buying.

|  | Water pressure and wave input Group: Core sensor Priority: Required Recommended item: Low-range submersible hydrostatic level transmitter, 4-20 mA preferred, target 0-1 mH2O or 0-2 mH2O Planning price: PHP 2,897-3,720 typical; lower listings around PHP 1,471 need strict verification Buy from or verify at: Lazada QDY30A / hydrostatic transmitter listings Approval note: Approve first. Main research sensor. Avoid high-range pump or automotive pressure sensors unless low-range water-level units cannot be verified. |
| --- | --- |

|  | Pressure signal interface Group: Interface Priority: Required interface Recommended item: ADS1115 16-bit ADC plus 150 ohm precision shunt, RC filter and protection parts Planning price: PHP 135-290 for ADS1115; add PHP 100-250 for shunt/protection Buy from or verify at: Lazada ADS1115 module listings Approval note: Approve with pressure sensor. ESP32 cannot directly read a 4-20 mA loop without interface parts. |
| --- | --- |

|  | Wind speed and direction Group: Core sensor Priority: Required Recommended item: Combined cup anemometer plus wind vane, RS485 / 0-5 V / 4-20 mA output preferred Planning price: PHP 1,204-1,639 low-cost; industrial units PHP 3,666-9,282 Buy from or verify at: Shopee anemometer listings; Lazada industrial anemometer listings Approval note: Approve low-cost combined speed and direction model first. If only speed is included, buy separate wind vane or mark wind direction as pending. |
| --- | --- |

|  | GPS position and security Group: Core support Priority: Required Recommended item: NEO-6M GPS module with ceramic antenna or equivalent GNSS board Planning price: PHP 299-497 common; higher listings up to PHP 1,143 Buy from or verify at: MakerLab PH / Lazada NEO-6M GPS listings Approval note: Approve. Provides exact position, drift/geofence evidence, timestamp support and anti-loss/security proof. |
| --- | --- |

|  | Battery and solar monitor Group: Power monitor Priority: Required support Recommended item: INA219 current and power monitor module; INA260 as upgrade option Planning price: PHP 83-180 for INA219; INA260 around PHP 593 Buy from or verify at: Shopee INA219, imBento Cebu, Lazada INA260 tag result Approval note: Approve INA219 first for low cost. It validates battery voltage, charging behavior, and power budget. |
| --- | --- |

|  | Rechargeable battery pack Group: Power system Priority: Required power Recommended item: Small 12 V LiFePO4 pack with built-in BMS, or protected 18650 pack for prototype validation Planning price: Prototype holders/cases from PHP 15-499; small 12 V LiFePO4 planning range PHP 1,500-3,500+ Buy from or verify at: Shopee 18650 holder/BMS listings; Lazada lithium/LiFePO4 battery listings Approval note: Add to approval list. Battery capacity must be sized after ESP32, LTE, GPS, sensors, and night light current are measured. |
| --- | --- |

|  | Solar panel Group: Power system Priority: Required power Recommended item: DIY mini solar panel or small solar cell module, about 3W-10W for the compact buoy; choose 5V, 6V, or 12V based on the final battery and charger design Planning price: DIY mini solar panels / small solar cell modules are commonly available online around PHP 188-249 for 3W-5W class; larger 12V kits cost more and may be too big for the compact buoy Buy from or verify at: Shopee/Lazada DIY mini solar panel, 5V/5W, 6V/5W, and small solar cell module listings Approval note: Add to approval list, but keep it physically small. DIY mini solar panels are acceptable for proposal validation if they are sealed or protected. Final wattage depends on measured daily load, available top-deck area, sun exposure, and whether the night beacon has its own solar panel. |
| --- | --- |

|  | Solar charger and battery protection Group: Power system Priority: Required power interface Recommended item: Solar charge controller matched to battery chemistry; include BMS for lithium/LiFePO4 packs Planning price: Small PWM controllers around PHP 189; CN3065/TP4056 class modules lower cost for single-cell prototypes Buy from or verify at: Shopee solar charge controller listings; CN3065/TP4056 module references Approval note: Required if solar charging is used. Do not connect solar panel directly to lithium battery without proper charger and protection. |
| --- | --- |

|  | Marine night light or beacon Group: Safety Priority: Required visibility/safety Recommended item: Waterproof solar marine beacon / buoy signal light, IP66-IP68 preferred, visible at night Planning price: Lazada/Shopee marine signal lights around PHP 269-781 for low-cost units; better marine lanterns cost more Buy from or verify at: Shopee solar marine navigation light; Lazada boat signal / solar beacon listings Approval note: Add for night visibility and safety. Prefer a self-contained solar beacon so it does not drain the main electronics battery. |
| --- | --- |

|  | Internet module Group: Communication Priority: Conditional communication Recommended item: ESP32 Wi-Fi for lab; A7670E LTE Cat-1 or Waveshare ESP32-S3 A7670E for field Planning price: A7670E core PHP 1,015-1,151; Waveshare ESP32-S3 A7670E PHP 2,614; T-SIM7600 class PHP 4,192 Buy from or verify at: Lazada A7670E / ESP32 SIM LTE listings Approval note: Keep as conditional. Use Wi-Fi for bench and near-shore tests; buy LTE only when the site has no stable Wi-Fi. |
| --- | --- |

|  | Tilt and motion Group: Optional Priority: Optional event/security Recommended item: MPU6050 / GY-521 accelerometer and gyroscope module Planning price: PHP 92-223 typical Buy from or verify at: Lazada MPU6050 listings Approval note: Optional. Adds movement or tilt event trigger. It does not replace the water pressure sensor. |
| --- | --- |

|  | Vibration and tamper Group: Optional Priority: Optional event/security Recommended item: SW-420 vibration sensor module Planning price: PHP 45-70 typical Buy from or verify at: e-Gizmo, MakerLab, Lazada sensor module tag results Approval note: Optional. Good for enclosure hit or tamper trigger; not reliable as wave measurement. |
| --- | --- |

|  | Lid-open tamper Group: Optional Priority: Optional event/security Recommended item: Reed switch or magnetic switch module Planning price: PHP 70-309 depending style Buy from or verify at: Lazada magnetic switch; local hobby electronics reed switch modules Approval note: Optional. Good for detecting if the enclosure is opened during field display or validation. |
| --- | --- |

# Alternatives and items not recommended as replacements

| Channel | Lower cost option | Better or safer option | Do not approve as replacement |
| --- | --- | --- | --- |
| Water pressure | Generic low-range 4-20 mA hydrostatic transmitter | Holykell HPT604 Type A or equivalent quoted industrial unit; RS485 Modbus if cable noise becomes a problem | Float switch, rain sensor, HC-SR04 ultrasonic sensor, automotive oil or fuel pressure sensor |
| Wind | Low-cost combined cup sensor and vane | Industrial RS485 combined sensor with IP65 or IP66 housing | Handheld anemometer or toy wind vane as final installed sensor |
| Power | Protected 18650 pack for bench validation | 12 V LiFePO4 pack with BMS and measured capacity | Loose cells with no BMS, no fuse, or no waterproof enclosure |
| Solar charging | Small solar charger module matched to the DIY panel and battery | Battery chemistry matched solar charger or MPPT controller | Direct solar panel to lithium battery connection |
| Night visibility | DIY mini solar panel / small solar cell module | IP66-IP68 marine beacon or self-contained solar buoy light | Indoor LED strip, flashlight, or non-waterproof decorative light |
| Internet | ESP32 Wi-Fi for lab | A7670E LTE Cat-1 or SIM7600 class board for field | Always-on every-second upload |
| Motion and tamper | SW-420 or reed switch | MPU6050 plus enclosure reed switch | Motion sensor as replacement for pressure sensor |

# Approval buying order

Use the existing ESP32 kit, breadboard, jumper wires, buttons, LEDs, and buck converters for bench validation only.

Buy or confirm the water pressure sensor and pressure interface first because this is the core measurement path.

Add GPS, INA219, and wind speed/direction after the pressure path is defined.

Measure ESP32, GPS, LTE/Wi-Fi, wind sensor, pressure interface, and beacon current before final battery sizing.

Choose the battery pack with BMS, matching solar charge controller, and solar panel after the load estimate is measured.

Use a self-contained solar marine beacon if possible so night visibility does not drain the main electronics battery.

Buy LTE only if the validation site has no reliable Wi-Fi. Keep LoRa as fallback only.

# Source references for price checking

Prices and availability can change. These links are included so the group can re-check the current seller page before purchase.

Pressure transmitter QDY30A listing: https://www.lazada.com.ph/videodetail/?video_id=8000144764410

Pressure transmitter 1 m 3 m 5 m listing: https://www.lazada.com.ph/videodetail/?video_id=8000146168991

Water level sensor tag result: https://www.lazada.com.ph/tag/tank-level-monitoring-system/

ADS1115 listing: https://www.lazada.com.ph/videodetail/?video_id=8000078224372

Shopee anemometer search: https://shopee.ph/list/anemometer

MakerLab Shopee anemometer listing: https://shopee.ph/Anemometer-Wind-Speed-Sensor-0-to-5V-Analog-RS485-i.18252381.40953188107

NEO-6M GPS listing: https://www.lazada.com.ph/videodetail/?video_id=8000133608045

Shopee INA219 listing: https://shopee.ph/INA219-GY-219-I2C-Bi-directional-DC-Current-Power-Supply-Sensor-Breakout-Module-for-Arduino-DIY-i.1063532622.25639962980

imBento INA219 listing: https://imbento.com/shop/product/439

18650 battery holder search: https://shopee.ph/list/18650%20battery%20holder

BeMuchSafer battery/BMS store: https://shopee.ph/bemuchsafer.ph

Shopee 12 V solar panel search: https://shopee.ph/list/12v%20solar%20panel

Lazada 10 W 12 V solar panel tag: https://www.lazada.com.ph/tag/solar-panel-10w-12v/

Solar marine navigation light listing: https://shopee.ph/love%2A-Solar-Powered-Marine-Navigation-Light-Waterproof-IP68-for-Fishing-Net-Buoys-i.135511368.51962091672

Lazada boat signal light tag: https://www.lazada.com.ph/tag/boatsignal-light-led/

A7670E listings: https://www.lazada.com.ph/tag/a7670e/

ESP32 LTE listings: https://www.lazada.com.ph/tag/esp32-sim/

MPU6050 listing: https://www.lazada.com.ph/videodetail/?video_id=8000145952787

SW-420 e-Gizmo: https://e-gizmo.net/gizmoshop/index.php?product_id=1611&route=product%2Fproduct

SW-420 MakerLab: https://makerlab.ph/products/vibration-sensor-sw-420

Magnetic switch listing: https://www.lazada.com.ph/videodetail/?video_id=8000131458280

5V/5W small solar panel listing: https://shopee.ph/GOLDEN-5V-5W-Small-Solar-Panel-With-USB-DIY-Monocrystalline-Silicon-Solar-Cell-Waterproof-Portable-Camping-Power-Panel-For-Mobile-Phone-PH-i.206104448.47456957544

Small solar panel search: https://shopee.ph/list/small%20solar%20panel

6V/5W solar panel tag: https://www.lazada.com.ph/tag/6v-5w-solar-panel/

DIY solar panel search: https://shopee.ph/list/diy%20solar%20panel

Small solar cell module search: https://shopee.ph/list/small%20solar%20cell