PROJECT FALCON DOCUMENT-CONTROL UPDATE — 10 SEPTEMBER 2026

STATUS OF THIS COPY: LEGACY / SUPERSEDED REFERENCE. Use BayStation.docx and docs/PROJECT_CONTEXT.md for the current thesis and architecture.

Current pressure-sensor baseline: Holykell HPT604 Type A, provisionally 0–2 mH2O vented gauge with 4–20 mA output, is the recommended long-duration deployment candidate. The exact order code, continuous-saltwater suitability, wetted materials, seal, cable and calibration must be confirmed before purchase. The protected 12 V loop requires a 150 ohm 0.1% shunt, input protection/filtering and a 3.3 V ADS1115 receiver. Bar02 is bench-only; its old I2C PCB and wiring are not fabrication-ready. No hardware is represented as purchased or field-validated.

# Project FALCON Sensor and Monitoring Baseline — v6.1

## Document Status

This document follows docs/PROJECT_CONTEXT.md v6.1. It separates the required Phase 1 baseline from supporting, health, security, and future functions. Physical integration and calibration remain pending.

## 1. Main Phase 1 Sensors

The required Phase 1 sensing functions are water pressure, GPS position, wind speed, and wind direction.

| No. | Sensor | Selected component | Interface | Primary measurements and purpose |
| --- | --- | --- | --- | --- |
| 1 | Water-pressure sensor | Bar02 or compatible; exact model to be verified | I2C | Raw/filtered pressure and calibrated estimated-wave support |
| 2 | GPS receiver | Exact model TBD | UART | Position, UTC, fix validity, satellite state, and persistent geofence |
| 3 | Wind-speed sensor | Exact model TBD | Pulse/GPIO | Local wind speed |
| 4 | Wind-direction sensor | Exact model TBD | Analog/ADC | Local wind direction |

Bar02 or a compatible pressure sensor is the primary wave-observation input. The system retains raw and filtered pressure, baseline, optional depth, estimated wave height, validity, timestamp/source, and calibration state.

## 2. Supporting Phase 1 Sensors and Monitors

These devices monitor energy, thermal condition, and additional environmental context. They support system reliability but are not all direct wave-height inputs.

| No. | Sensor or monitor | Selected component | Interface | Purpose |
| --- | --- | --- | --- | --- |
| 1 | Battery health monitor | Exact design TBD | Electrical interface TBD | Battery voltage/current/power and charge state |
| 2 | Solar health monitor | Exact design TBD | Electrical interface TBD | Solar charging and power status |
| 3 | Enclosure-temperature sensor | Exact model TBD | Interface TBD | Thermal status and warning input |
| 4 | Water-temperature probe | Sealed DS18B20 | One-Wire | Supporting water-temperature context |

## 3. Sensor Interface Component

The ADS1115 is part of the sensor interface but is not itself an environmental sensor.

| Component | Function |
| --- | --- |
| Adafruit ADS1115 analog-to-digital converter | Converts the analog wind-vane output into a digital value that the ESP32 can process |

## 4. Future Optional Sensors

The following devices are outside the Phase 1 baseline. They may be considered after project approval, funding, and successful validation of the focused system.

| Future sensor | Possible measurement or use | Minimum validation requirement |
| --- | --- | --- |
| BNO085 or other IMU | Optional motion research only; not required for wave estimation | Separate approved objective, calibration, and validation |
| Conductivity/salinity sensor | Electrical conductivity and salinity trend | Certified standard solution and temperature compensation |
| Turbidity sensor | Suspended-particle or water-clarity trend | Reference standards, fouling inspection, and optical cleaning |
| Dissolved-oxygen sensor | Dissolved oxygen condition | Air/water calibration and comparison with a reference meter |
| Chlorophyll-a or fluorometer sensor | Algal-concentration proxy | Laboratory or field reference comparison and optical maintenance |
| Additional water-temperature probes | Temperature at defined depths | Comparison with a traceable thermometer |
| Rain sensor | Local rainfall detection | Comparison with a reference rain gauge |
| UV sensor | Ultraviolet-exposure context | Reference-instrument comparison and exposure review |
| Water-current meter | Current speed and direction | Controlled-flow or reference-instrument comparison |
| Hydrophone | Passive underwater acoustic research | Sampling, storage, privacy, and research protocol |
| Waterline or level sensor | Local waterline relative to the buoy | Mounting-effect study and reference measurement |
| Leak or water-ingress sensor | Water inside the electronics enclosure | Not in current baseline; requires approved scope and controlled validation |
| Humidity/condensation sensor | Internal moisture and condensation risk | Not in current baseline; requires approved scope and controlled validation |
| Tamper or enclosure-open sensor | Unauthorized opening or maintenance event | Authorized-access and false-alarm testing |
| Weatherproof camera | On-demand visual inspection | Power, bandwidth, waterproofing, cybersecurity, and privacy testing |

The optional camera is intended for authenticated live viewing only. Routine recording and permanent video storage are not part of the proposed baseline. Computer vision would require a separate dataset, study objective, and validation.

## 5. Controllers and Computers

These components process sensor information but are not sensors.

| Component | Role |
| --- | --- |
| ESP32 DevKit | Initializes sensors, acquires readings, applies calibration, validates ranges, and creates timestamped telemetry |
| Orange Pi Zero 3 4GB | Receives ESP32 telemetry, stores SQLite records, performs deterministic local processing, and hosts the REST API and dashboard; optional AI remains isolated and non-blocking |
| Raspberry Pi 5 4GB | Conditional future edge-computer upgrade if Orange Pi benchmarks demonstrate insufficient performance |

## 6. Complete Data Flow

Required and supporting inputs → ESP32 acquisition, calibration, validation, and timestamping → USB serial/UART → Orange Pi Zero 3 4GB → SQLite and deterministic local processing → REST API → local dashboard. Internet synchronization and AI are optional and non-blocking.

## 7. Important Research Boundaries

- Current dashboard sensor values are simulated until physical integration is completed.

- Pressure-derived wave height must be labeled ESTIMATED and CALIBRATION REQUIRED until controlled validation is complete.

- All measurements require calibration, timestamps, units, validity, and fault states.

- Missing or invalid measurements must be shown as unavailable, not silently changed to zero.

- AI is optional and supporting; core monitoring, logging, security, and visualization must work without it.

- Future sensors require an approved research question, exact component selection, power and interface review, calibration method, maintenance plan, and field evidence.

- Project FALCON is a research and local decision-support prototype; it does not replace official PAGASA or other government monitoring and warning systems.

## 8. Summary

Phase 1 requires pressure, GPS, wind speed, and wind direction. A sealed DS18B20 provides supporting water temperature. Battery, solar, and enclosure temperature are health channels. Security uses persistent GPS geofence, a generic vibration/tamper input, an enclosure reed or limit switch, and a buzzer. BNO085, load-cell/HX711 mooring tension, salinity/conductivity, and mandatory AI are not required baseline functions.